APPLIED PHYSICS LAB

LAB 2

Department of Computer Sciences

## BAHRIA UNIVERSITY, ISLAMABAD

Name:

Enrollment No: 01-1342

Program: BS(CS)

Semester: 1st

Objectives:

- Measure the Least Count of the screw gauge.

- Find the zero error and make zero error correction.

- Measure the diameter of 5 objects and make a calculation table for each object using the Screw gauge.

Apparatus:

- Screw gauge

- Objects for measurement (Calculator’s lid, Register’s Cover, Computer Mouse’s Wire, Mobile Phone’s Cover,  Driving license Card)

Procedure:

- Determination of Pitch and Least Count (L.C):

  - We determined the pitch of  Screw Gauge by dividing the distance moved on the main (linear) scale by the total number of complete rotations given to the circular scale.

  - We counted the total number of divisions on the circular scale.

  - By using the following formula to calculate the least count of screw gauge.

Least Count=Pitch of the screwTotal number of circular scale divisions

- Determination of Zero Error:

  - We turn the ratchet so that anvil and spindle come in contact.

  - We observed that if the circular scale reading coincides with the  zero line on reference line on main scale.

  - If the zero does not coincides then check, Error is positive or negative by counting the division difference and multiplying it by the least count.

  - We write down the zero correction(c) which is equals to zero error, but with oppositive sign.

- Determination of Measurements of object:

  - We placed the object between the anvil and spindle.

  - We rotated the ratchet and make sure that the object is held firmly between the anvil and spindle.

  - We took the main scale reading(N).

  - We took the circular scale division (n) that coincides with the reference line on the main scale.

  - We multiple with circular scale reading(n) with least count(LC).   e.g: (n x LC)

  - In order to calculate the Observed Reading (R), we used the following formula.

R= N + (n x LC)

  - After that we apply the zero correction and calculated the mean of three reading of same object the get a reliable result.

Results and Calculations

Least Count calculation:

Least Count=Pitch of the screwTotal number of circular scale divisions =  0.550 = 0.01mm

Zero Error of the screw gauge:

Zero Error = (50 - 47) x LC = 3 x 0.01 = 0.03mm

Name of the Object =            CNIC Card           (Object 1)
Least Count =  0.01   mm =   0.001  cm

| Sr. no | Linear scale reading (N) (mm) | Circular scale reading |  | Total reading |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No of circular scale divisions on the reference line (n) | Value [n×(L.C)] mm | Observed R=N+n×(L.C) mm | Corrected D=R+c mm |
|  | 1 | 30 | 0.3 | 1.3 | 1.33 |
|  | 1 | 30 | 0.3 | 1.3 | 1.33 |
|  | 1 | 30 | 0.3 | 1.3 | 1.33 |

Mean =  1.33+1.33+1.333 = 1.33mm

Name of the Object =        Mouse Wire               (Object 2)
Least Count =  0.01  mm =    0.001   cm

| Sr. no | Linear scale reading (N) (mm) | Circular scale reading |  | Total reading |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No of circular scale divisions on the reference line (n) | Value [n×(L.C)] mm | Observed R=N+n×(L.C) mm | Corrected D=R+c |
|  | 3 | 17 | 0.17 | 3.17 | 3.2 |
|  | 3 | 17 | 0.17 | 3.17 | 3.2 |
|  | 3 | 17 | 0.17 | 3.17 | 3.2 |

Mean =  3.2 + 3. 2 + 3.23 = 3.2mm

Name of the Object =            Mobile Case           (Object 3)
Least Count =  0.01  mm =    0.001   cm

| Sr. no | Linear scale reading (N) (mm) | Circular scale reading |  | Total reading |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No of circular scale divisions on the reference line (n) | Value [n×(L.C)] mm | Observed R=N+n×(L.C) mm | Corrected D=R+c mm |
|  | 1.5 | 7 | 0.7 | 2.2 | 2.23 |
|  | 1.5 | 7 | 0.7 | 2.2 | 2.23 |
|  | 1.5 | 7 | 0.7 | 2.2 | 2.23 |

Mean =  2.33 + 2.23 + 2.233 = 2.23mm

Name of the Object =             Calculator’s Lid          (Object 4)
Least Count =  0.01  mm =    0.001   cm

| Sr. no | Linear scale reading (N) (mm) | Circular scale reading |  | Total reading |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No of circular scale divisions on the reference line (n) | Value [n×(L.C)] mm | Observed R=N+n×(L.C) mm | Corrected D=R+c mm |
|  | 2.5 | 34 | 0.34 | 2.84 | 2.87 |
|  | 2.5 | 34 | 0.34 | 2.84 | 2.87 |
|  | 2.5 | 34 | 0.34 | 2.84 | 2.87 |

Mean =  2.87 + 2.87 + 2.873 = 2.87mm

Name of the Object =          Register Cardboard             (Object 5)
Least Count =  0.01  mm =    0.001   cm

| Sr. no | Linear scale reading (N) (mm) | Circular scale reading |  | Total reading |  |
| --- | --- | --- | --- | --- | --- |
|  |  | No of circular scale divisions on the reference line (n) | Value [n×(L.C)] mm | Observed R=N+n×(L.C) mm | Corrected D=R+c mm |
|  | 0.5 | 23 | 0.23 | 0.73 | 0.76 |
|  | 0.5 | 23 | 0.23 | 0.73 | 0.76 |
|  | 0.5 | 23 | 0.23 | 0.73 | 0.76 |

Mean =  0.76 + 0.76 + 0.763 = 0.76mm

Conclusion:

In this experiment, we learned how to properly use a micrometer screw gauge to take accurate small-scale measurements. First, we calculated the least count and checked for any zero error so we could correct our findings. By taking three separate readings for each of the five objects. Finally, we applied the zero correction to find the exact mean diameter for every object, which proved how precise a screw gauge is compared to standard measuring tools.